Shenzhen Baichuang New Energy Industrial Co,.Ltd

Specifications

Product Name：Solar Wall Light with Flame Effect

No：CP22

September 15,2017

| prepared by | Audit |  |
| --- | --- | --- |
| Shangru Wu | Shili Zheng |  |

- Catalogue

- 1、product picture- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - 3

- 2、Product features- - - - - - - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - 3

- 3、Application- - - - - - - - - - - - - - - - - - - - - - - - -- - - - - - - - - - - - - - - - - - - - 4

- 4、Technology parameters- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - 4

- 5、Function - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - 5

- 6、Package- - - - - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - 5

- 7、Size and weight - - - - - - - - - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - 5

- 8、Installation- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - 6

- Product picture               2、 Product features

- Solar energy/wireless

- Easy installation

- Use for lawn and wall

- Automatic light sensor

- Automatic human sensor

- Smart chips protect the battery

- Long lifespan（25000h）

- Branded LED chip

- Energy saving

- 3、Application：

- villa、yard、lawn.

- Landscape lighting

- Technology parameters

| Product type | BC-CP12 |
| --- | --- |
| Working power | 2.0 W |
| Voltage | DC 3.7V |
| Current when turn off | <3uA |
| Brightness | 10 lux |
| Distance induction | 5~8m |
| Installation height | 2-4m |
| LEDs | 2835 12pcs |
| CCT | 6500-7500K |
| Solar panel | polycrystal 1.8 W 5.5V |
| Battery | Li-18650 2000mAH |
| Working Hours | 8~12H |
| Lifespan | 25000H |
| Lighting times | 100000 |
| Working Temperature | -20℃…+45℃ |
| Waterproof | IP65 |
| Mercury content | 0.0 mg |

- Function

- 1、Three modes（Enter working mode when the light is low）：

First mode：The light turn on when people coming, turn off when people gone（The light on when induction human activities, The lights off after 15s of non sense of human active）

Second modes：The light turn on when people coming,the light glimmer when people gone（The light on when induction human active, The lights off after 15s of non sense of human active）

Third mode：Long and low light（The light turn on during the night, the light turn off during the day）

2、Operating steps：

- Long press the button 3S when the light off, and the light flashes once when entering mode 1.Press the button again and the light flashes twice to enter mode 2.Press the button again and the light flashes thrice down into mode 3.Press shut down again.

- 6、Packing

| Box Size | 14*10*20cm |
| --- | --- |
| Weight | 0.39 kg |
| Pcs/carton | 20pcs |
| Carton Size | 57.5*51.5*22cm |
| Gross Weight/carton | 9.0 kg |
| Net Weight/carton | 7.8 kg |

- 7、Size and weight

| Length*Width*heigh | 236.9*92.5*135.2mm |
| --- | --- |
| weight | 0.32 kg |

8、Installation

一、Wall installation

- 1、Punch holes in the wall and plug the glue plug into the holes

2.Install the base on the wall and fix it with screws.

3.The lamp body is mounted on the bottom seat and fixed with screws.

二、Insert installation

1.The lamp body is mounted on the bottom seat and fixed with screws.

2、Stick the floor into the ground and then onto the lawn. The solar panels must be in the direction of the sun.

Remark:Shenzhen Baichuang New Energy Industrial Co,.Ltd reserves the final interpretation. Due to the continuous upgrading of the product, the specifications are would change without notice, please contact us with the latest product information.

Remark:Shenzhen Baichuang New Energy Industrial Co,.Ltd reserves the final interpretation. Due to the continuous upgrading of the product, the specifications are would change without notice, please contact us with the latest product information.