便携式多功能储能系统

Portable Energy Storage System

PES-A9操作说明书

PES-A9 Operating Manual

系统简介：

System Introduction：

PES-A9 便携式多功能储能系统，结合太阳能充电、蓄电、放电控制管理为设计基础，内置高容量、高性能磷酸铁锂电池，采用当前先进汽车动力锂电池工艺制造，具有多重安全保护，输出能量强劲，提供交流电压输出、直流不同电压输出模式，方便不同用电器供电，内置蓝牙音箱、强光手电筒、两档台灯照明、SOS闪光模式，适用于家用电源、紧急应急电源、支持危险救难、支持缺乏电力之区域、提供野营或旅行电源等使用模式。

PES-A9 portable multi-functional energy storage system, combined with solar charging, storage, discharge control management as the design basis, built-in high-capacity, high-performance lithium iron phosphate batteries, using the current advanced automotive power lithium battery manufacturing process, with multiple security protection, strong output energy, provide AC voltage output, DC Different voltage output two modes, convenient for different electrical power supply, built-in Bluetooth speaker, flashlight, two-stage lamp lighting, SOS flash mode, suitable for household power, emergency power supply, support dangerous rescue, support the lack of power areas, provide camping or travel power mode

安全提示

Security Tips

为确保您安全、正确使用本产品，请遵守下列事项：

Safety note and the precautions:

- 产品请放置于儿童触摸不到的地方。 
Please place the product in a place where children cannot touch it.

- 请使用原厂或正规渠道购买的配件。

Please use the accessories purchased from the original or regular channels.

- 请勿超过额定负载使用本产品。

Do Not exceed the rated load for the use of this product.

- 请在通风良好环境下使用，切勿在以下环境中保管或使用本产品：

Use in a well ventilated environment, do not keep in the following environment or use of this product:

a. 有可燃性气体、腐蚀性物质、大量灰尘的场所

A place with flammable gases, corrosive substances, and large amounts of dust

b. 异常高温或低温(40℃以上或0℃以下)，高湿(90％以上)的场所

abnormal high or low temperature (40 DEG C or below 0 DEG C), high humidity (more than 90%) place

c. 有阳光直射或接近加热器具的场所

A place with direct sunlight or close to a heating device

d. 有剧烈振动的场所

A place with violent vibration

- 电量低于30%或三个月未使用本产品请及时充电。

the amount of electricity is less than 30% or three months without

the use of this product, please charge it in time.

- 产品内装有大容量磷酸铁锂电池，非专业人士不可打开机壳，否则会有触电、起火危险.

The inside of the product has a large capacity lithium battery, non professionals 	can not open the shell, otherwise there will be electric shock, fire danger.

如有冒烟、失火，请使用干粉灭火器，使用液体灭火器有触电危险。

If there is smoke and fire, please use the dry powder fire extinguisher and the use 		of the liquid fire extinguisher to be in danger of electric shock.

外观介绍（一）

Appearance introduced（1）

状态指示灯

电量指示灯

LED照明区

直流开关 DC On-off

系统开关 System On-off

交流开关 AC On-off

两路DC12V输出

Two DC 12V Output

两路USB A输出

Two USB A Output

两路TYPE C输出

Two Type C Output

外观介绍（二）

Appearance introduced（2）

喇叭

speaker

蓝牙音箱按键

Sound switch

AC输出

AC output

TF卡座

TF card seat

手电筒灯

Flashlight

充电口

Charging port

操作说明(一)

- 单击“POWER”键，电量灯亮10S，显示电量信息

- 长按“POWER”键，红灯亮起后表示系统正常，再次长按后熄灭，表示系统已关机

- 在开机状态下，单击“DC ON”，DC蓝灯亮，表示USB A&TYPE C口输出打开,可以给手机等5V设备供电,同时通过不同按键方式可控制手电筒及台灯照明模式

3.1双击开启台灯照模式，在开启台灯照明后，单击DC ON，台灯亮灯模式：亮高度  SOS 循环，再次双击关闭台灯

3.2 长按3秒，打开强光手电筒，再次长按3秒，关闭手电筒

注:在无负载情况下,30-60秒内会关闭输出

- 在开机状态下，单击“AC ON”， AC 绿灯亮，表示AC口输出打开,可以给设备提供交流供电,再次单击后, AC 灯灭,表示输出关闭AC输出最大功率300W,超出350W时,系统会进入保护状态,需要移除负载重新开机

- DC 12V输出口支持即插即用,可支持36W内负载

Operating instructions( 1 )

- Click the "POWER" button, and the power lamp lights up 10S to display the power information.

- Press the "POWER" button for a long time. When the red light is turned on, the system is normal. When the red light is pressed again, the system is turned off.

- In the boot state, click the "DC ON", DC blue light, said USB A&TYPE C output port open, can supply to the mobile phone and other 5V devices, while controlling the flashlight and lamp lighting mode by different key mode

3.1  Double click open lamp lighting mode, lamp lighting in the open, click the DC ON lamp light pattern: high bright  SOS cycle, double click again to close the table lamp

3.2  press 3 seconds to open the flashlight flash light, and then press the flashlight for 3 seconds.

Note: in the case of no load, the output will be closed within 30-60 seconds

- In boot state, click "AC ON" and the AC green light is on, indicating that the output of AC port is on, which can provide AC power to the equipment. When clicked again, the AC light is off, indicating that the output is off.

AC output maximum power 300W, over 350W, the system will enter the protection state, need to remove the load reboot

- DC 12V export support plug and play,can support within 36W load

操作说明(二)

Operating instructions( 2 )

- 蓝牙操作说明：

Bluetooth operation instructions:

  - ：开、关蓝牙音箱，长按  约3秒，开启蓝牙音箱功能，听到提示音后，开启成功;关机:蓝牙音箱开机状态长按  约3秒,蓝牙音箱关闭;

Turn on and off the Bluetooth speaker, press about 3 seconds long, turn 	on the Bluetooth speaker function, after hearing the prompt, turn on 	successfully.

  - 蓝牙配对：进入需要和蓝牙音箱连接设备的“设置”，找到“蓝牙”选项，点击“蓝牙”以启动蓝牙功能，扫描连接设备，找到“A9”设备，配对成功后可进行音乐播放,下次蓝牙音箱开机优先自动上次连接过的设备.不需手动连接.

Bluetooth Pairing: Enter the "Settings" for devices that need to be 	connected to the Bluetooth speaker, find the "Bluetooth" option, click 	"Bluetooth" to start the Bluetooth function, scan the connection device, 	and find the "A9" device. When the pairing is successful, the music can 	be played. 	Next time the Bluetooth speaker boots, the device that was 	automatically 	connected is priority. No manual connection is required.

  - ：    短按播放上一曲，长按>1秒音量递加

short press play the last song, long press > 1 second volume added.

  - ：    短按播放键暂停

short press play the last song, long press > 1 second volume added.

  - ：    短按播放下一曲，长按>1秒音量递减

Short Click to play the next song, long press > 1 second volume 	reduction.

  - 当  TF卡插入卡槽时，自动播放TF卡内音乐，可通过按键控制音乐

When the TF card into the card slot, TF card automatically play music, music can be controlled by buttons

- 充电说明

PES-A9支持15V4A适配器充电，请使用原装充电器，同时支持18V 最大150W太阳能充电.

PES-A9 supports the 15V4A adaptor, please use the original charger, 	while supporting 18V 150W maximum solar charging.

| 电气参数 Electrical parameters | 规格 Specifications |
| --- | --- |
| 型号 Model | PES-A9 |
| 电池容量 Power capacity | 307WH±5% |
| AC输出电压 AC Output voltage | 220VAC 50HZ |
| AC额定功率 AC Power Rating | 额定功率300瓦 最大:350瓦 Rated power：300W， MAX：350W |
| 输出波形 Output Waveform | 修正波 Amendment Wave |
| DC 12V输出功率 DC 12V Output Rating | 12V 3A |
| USB输出功率 USB Output Rating | 5V/2.1A |
| TYPE C输出 TYPE C Output | 5V/2.1A |
| 蓝牙音箱功率 Audio power | 3W |
| 音频带宽 Audio bandwidth | 20HZ-20KHZ |
| 信噪比 Signal-to-noise ratio | ≥90db |
| 失真度 Distortion degree | ≤1% |
| 工作范围 Work scope | 10米 /10 meters |
| 蓝牙规范 bluetooth profile | Bluetooh V4.0 |
| 适配器功率 Adapter power | 15V/4A |
| 太阳能板功率 Solar charging Voltage | 18V MAX：120W |
| 过温保护 Over-Temperature Protection | 过温时关闭输出,恢复后重新启动 Turn off the output at over temperature and restart after recovery |
| 过载保护 Overpower Protection | 负载过载时关闭输出,恢复后重新启动 When load is overloaded, the output is closed and restarted after recovery. |
| 短路保护 Short-circuit Protection | AC输出短路时关闭输出,恢复后重新启动 When AC output short circuit, turn off output and restart after recovery |
| 重量 Weight | 4.80KG |
| 尺寸 Dimension | 274.5X261X152.5MM |

使用时间:

Uses time:

| 产品 Product | 时间 Uses time |
| --- | --- |
| 3000mAH手机电池 Mobile phone battery | 充电30次 Charging 30 times |
| 笔记本Laptop | 3-6H |
| 台式机Desktop | 1-1.5H |
| 50W电风扇 50W electric fan | 5H |
| 32寸液晶电视32 inch LCD TV | 4-6H |
| 12V/3W LED灯 | 10-100H |
| 25W电灯泡25W Bulb | 10-12H |
| 冰箱 Refrigerator | 6-12H |

保修卡

感谢您使用本产品！我公司为您提供自购买之日起12个月内的保修服务。需要保修时请出示本保修卡，并请详细填写相关内容。

保修范围：

1.在正常使用状态下出现的质量问题

2.如因操作失误造成对产品的损害不在保修范围内

3.自行拆机后不予以保修

不在保修范围内的产品，我司可提供维修服务，但维修费用需由客户承担

| 产品型号 |  | 出厂编号 |  |
| --- | --- | --- | --- |
| 购买日期 |  | 维修日期 |  |
| 客户姓名 |  | 联系电话 |  |
| 客户地址 |  |  |  |
| 销售单位 |  |  |  |
| 产品异常描述 |  |  |  |

Warranty card

Thank you for using our product! Our company will provide you with warranty services within 12 months from the date of purchase. Please show this warranty card when you need warranty, and please fill in the relevant contents in detail.

scope of warranty：

1.Quality problems in normal use

2.If the operation is wrong, the damage to the product is not within the scope of warranty.

3.No warranty after self dismantling.

We can provide repair service without warranty within the scope of warranty, but the cost of maintenance shall be borne by the customer.

| Product model |  | Factory number |  |
| --- | --- | --- | --- |
| Dateof purchase |  | Maintenance date |  |
| Customer name |  | Contact number |  |
| Customer address |  |  |  |
| point of sale |  |  |  |
| Product exception description |  |  |  |